PS-CSP TM/TC Definition

Current PSC_CSP implementation basis, generated 2026-08-14. Google Docs default preset.

This document summarizes the implemented CSP/RS485 telecommand and telemetry interface for the PSC working tree. USART1 uses binary CSP/KISS framing; the legacy ASCII $iGRVT50 line protocol is not used on this link.

# CSP definition

| Address | Node | Implementation note |
| --- | --- | --- |
| 0x0A | OBC | Master/requester. PSC responses are only sent to accepted OBC requests. |
| 0x0B | RPi | Reserved from PS-CSP allocation; not used by the current PSC firmware. |
| 0x10 | PS / PSC | Local PSC responder address. |

| Item | Current value | Note |
| --- | --- | --- |
| Physical | RS485 over USART1 | No external DE/RE pin is controlled by PSC. |
| Baud | 921600 bps, 8N1 | Matches current GUI default. |
| Frame | CSP v1.x 32-bit header + KISS + CRC32C | Multi-byte PSC payload fields are big-endian. |
| Behavior | Responder only | PSC transmits only as a response to OBC requests. |
| Control loop | 100 Hz / 10 ms | Thruster and PAR services advance on this period. |

# CSP PS definition

| Port | Service | Handler / payload | Status |
| --- | --- | --- | --- |
| 1 | CSP_PING | Standard libcsp service. Current GUI sends payload "PSC-PING" and expects the same payload echoed. | Implemented |
| 4 | CSP_REBOOT | Standard libcsp service. Request payload is magic word 0x80078007. No PSC common status payload. | Implemented |
| 10 | PS_COMMAND | PSC command TC with common PSC request/response header. | Implemented |
| 11 | PS_TELEMETRY | Sensor TM and solvalve state TM request port. | Implemented |
| 12 | PS_DIAGNOSTICS | Health/debug TM request port. | Implemented |
| 13-15 | Reserved | Earlier PDF sections are not bound by the current PSC_CSP firmware. | Not implemented |

# Common PSC payload header

The common PSC header is used on ports 10, 11, and 12 only. CSP_PING and CSP_REBOOT use the standard libcsp service payloads.

| Direction | Offset | Field | Size | Description |
| --- | --- | --- | --- | --- |
| Request | 0 | version | 1 | Protocol version, currently 0x01. |
| Request | 1 | opcode | 1 | Port-local command or TM request opcode. |
| Request | 2-3 | transaction_id | 2 | Big-endian transaction identifier echoed in response. |
| Response | 0 | version | 1 | Protocol version, currently 0x01. |
| Response | 1 | opcode | 1 | Echo of request opcode. |
| Response | 2-3 | transaction_id | 2 | Echo of request transaction_id. |
| Response | 4 | status | 1 | Common PSC status code. |
| Response | 5 | detail | 1 | Status-specific detail; 0 when not used. |

| Code | Status | Meaning |
| --- | --- | --- |
| 0x00 | OK | Request accepted and applied, or TM response valid. |
| 0x01 | BAD_VERSION | Payload version does not match 0x01. |
| 0x02 | BAD_LENGTH | Payload length does not match the opcode contract. |
| 0x03 | BAD_OPCODE | Opcode is not valid for the destination port. |
| 0x04 | INVALID_ARGUMENT | Field value or reserved bits are invalid. |
| 0x05 | INVALID_STATE | Command is valid but not allowed in the current state/mode. |
| 0x06 | APPLY_FAILED | Domain layer failed to apply the request. |
| 0x07 | INTERNAL_ERROR | Internal decode/encode failure. |
| 0x08 | BUSY | Reserved for future busy handling. |
| 0xFF | DROP | Packet should be dropped with no response when classified as invalid peer/port/frame. |

# Mode codes and TC applicability

| Code | Mode | Note |
| --- | --- | --- |
| 0x00 | init_mode | Initial mode. |
| 0x01 | normal_mode | Normal command mode. |
| 0x02 | run_mode | Thruster sequence active mode. |
| 0x03 | diagnostic_mode | Direct HPV control is allowed only in this mode. |

| TC / service | Accepted mode | Application rule |
| --- | --- | --- |
| CSP_PING | All modes | Standard CSP service; response echoes request payload. |
| CSP_REBOOT | All modes | Standard CSP reboot service with magic word. |
| SET_OUTPUTS | All modes | LPV, heater, and SP apply directly. HPV bits are applied only in diagnostic_mode. |
| SET_LPV_OUTPUTS | All modes | LPV-only control; does not touch HPV, heater, or SP state. |
| SET_MODE | StateMachine policy | Payload is accepted if the requested mode is in range and StateMachine_RequestMode accepts the transition. |
| THRUSTER_START | Normal only | Accepts burn_time_ms, requests run_mode, starts the configured HPV/SP sequence. |
| PAR_START | Normal only | Starts PAR routine. Once started, PAR service continues regardless of later mode changes. |
| PAR_STOP | All modes | Stops PAR routine for safety. |
| GET_SENSOR_SNAPSHOT | All modes | Returns sensor TM. |
| GET_SOLVALVE_STATE | All modes | Returns solvalve/heater/SP state TM. |
| GET_HEALTH | All modes | Returns link health and up to four queued debug messages. |

# 10 - PS_COMMAND

| Opcode | Command | Req len | Payload after common header | Response | Mode rule |
| --- | --- | --- | --- | --- | --- |
| 0x01 | SET_OUTPUTS | 10 | lpv_mask[2], hpv_mask[1], heater_mask[1], spark_on[1], reserved[1] | Common status, 6 bytes | All modes; HPV applies only in diagnostic_mode. |
| 0x02 | SET_MODE | 5 | mode[1] | Common status, 6 bytes | StateMachine transition policy. |
| 0x03 | THRUSTER_START | 8 | burn_time_ms[4] | Common status, 6 bytes | Normal only. |
| 0x04 | PAR_START | 4 | No extra payload | Common status, 6 bytes | Normal only. |
| 0x05 | PAR_STOP | 4 | No extra payload | Common status, 6 bytes | All modes. |
| 0x06 | SET_LPV_OUTPUTS | 6 | lpv_mask[2] | Common status, 6 bytes | All modes; LPV only. |

| Command | Offset | Field | Size | Meaning |
| --- | --- | --- | --- | --- |
| SET_OUTPUTS | 4-5 | lpv_mask | 2 | bit0=LPV1 through bit11=LPV12. Valid mask 0x0FFF. |
| SET_OUTPUTS | 6 | hpv_mask | 1 | bit0=HPV1 through bit7=HPV8. Applied only in diagnostic_mode. |
| SET_OUTPUTS | 7 | heater_mask | 1 | bit0=HTR1 through bit3=HTR4. Valid mask 0x0F. ON means 100 percent duty. |
| SET_OUTPUTS | 8 | spark_on | 1 | 0=SP off, 1=SP on. Other values invalid. |
| SET_OUTPUTS | 9 | reserved | 1 | Must be 0. |
| SET_LPV_OUTPUTS | 4-5 | lpv_mask | 2 | LPV bit mask only. No side effect on HPV/heater/SP. |
| SET_MODE | 4 | mode | 1 | 0=init, 1=normal, 2=run, 3=diagnostic. |
| THRUSTER_START | 4-7 | burn_time_ms | 4 | Burn time in milliseconds. 0 is invalid. |

## Thruster sequence

| Action | Time reference | Delay / duration | Debug event |
| --- | --- | --- | --- |
| Pre-run ignition condition check | Immediately after THRUSTER_START acceptance | 0 ms | THRUSTER_CHECK.PRE_RUN_CHECK |
| Enter run_mode | After pre-run check passes | Immediate request to run_mode | Mode appears in later TM/debug. |
| SP ON | THRUSTER_START acceptance | 500 ms | THRUSTER.SP_ON |
| HPV1 ON | THRUSTER_START acceptance | 1000 ms | THRUSTER.HPV1_ON |
| HPV2 ON | THRUSTER_START acceptance | 2000 ms | THRUSTER.HPV2_ON |
| Run monitor | While sequence is active | Every 1000 ms | THRUSTER_CHECK.RUN_MONITOR_1HZ |
| SP OFF | SP ON timing | 3000 ms duration; off at 3500 ms elapsed | THRUSTER.SP_OFF |
| HPV1 OFF | burn_time_ms | 500 ms after burn end | THRUSTER.HPV1_OFF |
| HPV2 OFF | burn_time_ms | 2000 ms after burn end | THRUSTER.HPV2_OFF |
| Return normal_mode | After HPV1 OFF, HPV2 OFF, and SP OFF are complete | Immediate | Reflected in subsequent TM. |

## PAR routine

| Item | Current behavior |
| --- | --- |
| Start condition | PAR_START is accepted only in normal_mode. |
| Stop condition | PAR_STOP is accepted in all modes. |
| Runtime | After PAR_START, ParRoutine_Service10ms is called every 10 ms while active, regardless of later mode. |
| Debug | PAR.PAR_ROUTINE_1HZ is queued every 1000 ms while active; PAR_START and PAR_STOP are also queued. |

# 11 - PS_TELEMETRY

| Opcode | TM request | Req len | Response length | Purpose |
| --- | --- | --- | --- | --- |
| 0x01 | GET_SENSOR_SNAPSHOT | 4 | 66 | Pressure transducer and thermocouple measurement snapshot. |
| 0x02 | GET_SOLVALVE_STATE | 4 | 16 | LPV, HPV, heater, and spark plug state snapshot. |

## Sensor TM response

| Offset | Field | Size | Description |
| --- | --- | --- | --- |
| 0-5 | common response | 6 | version, opcode, transaction_id, status, detail. |
| 6-9 | sample_time_ms | 4 | Snapshot time/tick in milliseconds. |
| 10 | current_mode | 1 | Current state-machine mode. |
| 11 | requested_mode | 1 | Requested mode from state machine. |
| 12-13 | validity_mask | 2 | Sensor validity bits. bit0-bit8=PT1-PT9, bit9-bit12=TC1-TC4. |
| 14-49 | pt_millivolt[9] | 36 | Nine signed int32 values in millivolts. |
| 50-65 | tc_microvolt[4] | 16 | Four signed int32 values in microvolts. |

## Solvalve TM response

| Offset | Field | Size | Description |
| --- | --- | --- | --- |
| 0-5 | common response | 6 | version, opcode, transaction_id, status, detail. |
| 6-9 | sample_time_ms | 4 | Snapshot time/tick in milliseconds. |
| 10 | current_mode | 1 | Current state-machine mode. |
| 11-12 | lpv_mask | 2 | bit0=LPV1 through bit11=LPV12. |
| 13 | hpv_mask | 1 | bit0=HPV1 through bit7=HPV8. |
| 14 | heater_mask | 1 | bit0=HTR1 through bit3=HTR4. |
| 15 | spark_on | 1 | 0=SP off, 1=SP on. |

# 12 - PS_DIAGNOSTICS

| Opcode | TM request | Req len | Response length | Purpose |
| --- | --- | --- | --- | --- |
| 0x01 | GET_HEALTH | 4 | 106 | Link health, last error, counters, and queued debug messages. |

## Health TM response

| Offset | Field | Size | Description |
| --- | --- | --- | --- |
| 0-5 | common response | 6 | version, opcode, transaction_id, status, detail. |
| 6-9 | uptime_ms | 4 | Current uptime in milliseconds. |
| 10 | current_mode | 1 | Current state-machine mode. |
| 11 | link_state | 1 | RS485/CSP link state reported by the link layer. |
| 12 | last_error | 1 | Most recent link/service error code. |
| 13 | debug_count | 1 | Number of valid debug slots, capped at four. |
| 14-61 | debug_messages[4] | 48 | Each debug message is 12 bytes. |
| 62-105 | counters[11] | 44 | Eleven uint32 link/service counters. |

| Debug field | Size | Description |
| --- | --- | --- |
| sequence | 4 | Monotonic debug message sequence. |
| elapsed_ms | 4 | Elapsed time for the event source. |
| source | 1 | 1=THRUSTER, 2=THRUSTER_CHECK, 3=PAR. |
| event | 1 | Source-specific event code. |
| mode | 1 | Mode sampled when the debug message was queued. |
| reserved | 1 | Reserved, currently 0. |

| Source | Event code | Event name |
| --- | --- | --- |
| THRUSTER | 0 | HPV1_ON |
| THRUSTER | 1 | HPV2_ON |
| THRUSTER | 2 | SP_ON |
| THRUSTER | 3 | HPV2_OFF |
| THRUSTER | 4 | HPV1_OFF |
| THRUSTER | 5 | SP_OFF |
| THRUSTER_CHECK | 0 | PRE_RUN_CHECK |
| THRUSTER_CHECK | 1 | RUN_MONITOR_1HZ |
| PAR | 0 | PAR_ROUTINE_1HZ |
| PAR | 1 | PAR_START |
| PAR | 2 | PAR_STOP |

| Index | Counter name | Source field |
| --- | --- | --- |
| 0 | uart | uart_errors |
| 1 | dma | dma_errors |
| 2 | tx_to | tx_timeouts |
| 3 | tx_fail | tx_failures |
| 4 | proto | protocol_errors |
| 5 | drop | stream_dropped_bytes |
| 6 | hwm | stream_high_watermark |
| 7 | disc | stream_discontinuities |
| 8 | rec_try | recovery_attempts |
| 9 | rec_ok | recovery_successes |
| 10 | rec_fail | recovery_failures |

# Bit packing summary

| Field | Valid bits | Meaning |
| --- | --- | --- |
| lpv_mask | bit0-bit11 | LPV1-LPV12. Valid mask 0x0FFF. |
| hpv_mask | bit0-bit7 | HPV1-HPV8. |
| heater_mask | bit0-bit3 | HTR1-HTR4. Valid mask 0x0F. |
| spark_on | bit/value 0 or 1 | 0=SP off, 1=SP on. |
| sensor validity | bit0-bit8 | PT1-PT9 valid. |
| sensor validity | bit9-bit12 | TC1-TC4 valid. |

# Implementation notes

| Topic | Current implementation |
| --- | --- |
| ASCII protocol | USART1 ASCII $iGRVT50 command/TM is not used after CSP conversion. |
| HPV direct control | HPV bits in SET_OUTPUTS are ignored outside diagnostic_mode. Thruster sequence controls HPV1 and HPV2 internally. |
| Heater control | HTR1-HTR4 use Heater_SetDuty with 100 percent for ON and 0 percent for OFF. |
| Spark plug | SP is PC5 GPIO ON/OFF. It is not PWM controlled. |
| Debug visibility | Debug messages are returned only inside GET_HEALTH responses. GUI 1 Hz auto health polling must be enabled to see periodic debug output. |