PSC-CSP Interface Control Document

| Field | Value |
| --- | --- |
| Revision | Draft A |
| Date | 2026-08-24 |
| Branch | PSC_CSP |
| Code basis | commit ffe00d6 plus current working-tree protocol changes |
| Target reader | OBC CSP/RS485 software implementer |

Editable OBC implementation ICD generated from the current PSC-CSP Markdown specification.

# 1. Scope

This ICD defines the CSP/RS485 interface between OBC and PSC.

PSC is a CSP responder on RS485. OBC is the bus master and initiates every transaction. PSC transmits only as a response to an OBC request, except the CSP reboot command where no application-level response shall be expected. SET_LPV_OUTPUTS success telemetry is request-triggered but broadcast on the CSP bus.

USART1 no longer uses the legacy ASCII $iGRVT50 protocol.

# 2. Physical And Link Layer

| Item | Value |
| --- | --- |
| Physical link | RS485 UART |
| Baudrate | 921600 bps |
| UART format | 8N1 |
| Wire byte size | 10 bits including start/stop |
| PSC behavior | Responder only |
| Recommended OBC transaction model | One outstanding request at a time |
| Suggested response timeout | 500 ms |
| Suggested retry count | up to 5 retries |

# 3. CSP Address And Ports

| Node | CSP Address |
| --- | --- |
| OBC | 0x0A |
| PSC | 0x10 |
| Broadcast | 0x1F |

| Port | Name | Direction | Description |
| --- | --- | --- | --- |
| 1 | CSP_PING | OBC -> PSC -> OBC | CSP standard ping echo |
| 4 | CSP_REBOOT | OBC -> PSC | CSP standard reboot |
| 10 | COMMAND | OBC -> PSC -> OBC/broadcast | PSC telecommands |
| 11 | TELEMETRY | OBC -> PSC -> OBC | PSC telemetry requests |
| 12 | DIAGNOSTICS | OBC -> PSC -> OBC | PSC health request |

OBC shall use source ports in the dynamic range 14..63. PSC responses use source address 0x10, destination address 0x0A, source port equal to the requested destination port, and destination port equal to the OBC request source port. Exception: successful SET_LPV_OUTPUTS returns the same Sensor Snapshot payload to destination address 0x1F while preserving source/destination port mapping.

# 4. CSP Frame Encoding Over RS485

### 4.1 CSP ID

CSP ID is transmitted as a 32-bit big-endian value before CSP data.

| Bits | Field | Width | Current OBC Request Value |
| --- | --- | --- | --- |
| 31..30 | Priority | 2 | 2 |
| 29..25 | Source address | 5 | 0x0A |
| 24..20 | Destination address | 5 | 0x10 |
| 19..14 | Destination port | 6 | target port |
| 13..8 | Source port | 6 | 14..63 |
| 7..0 | Flags | 8 | 0x00 |

Packing formula:

uint32_t csp_id =
    ((priority & 0x03) << 30) |
    ((source & 0x1F) << 25) |
    ((destination & 0x1F) << 20) |
    ((destination_port & 0x3F) << 14) |
    ((source_port & 0x3F) << 8) |
    (flags & 0xFF);

### 4.2 CRC32C

Each RS485 CSP packet appends a 4-byte CRC32C to the CSP data. The CSP header is not included in CRC calculation.

| Item | Value |
| --- | --- |
| Algorithm | CRC32C |
| Initial value | 0xFFFFFFFF |
| Reflected polynomial | 0x82F63B78 |
| Final XOR | 0xFFFFFFFF |
| CRC coverage | CSP data/application payload only |
| CRC byte order | Big endian |

### 4.3 KISS Framing

Raw bytes before KISS escaping:

CSP_ID_BE32 | CSP_DATA | CRC32C_BE32

KISS frame:

0xC0 | 0x00 | escaped(CSP_ID_BE32 | CSP_DATA | CRC32C_BE32) | 0xC0

Escape rules:

| Raw byte | Escaped bytes |
| --- | --- |
| 0xC0 | 0xDB 0xDC |
| 0xDB | 0xDB 0xDD |

The second byte after the opening 0xC0 shall be KISS TNC data byte 0x00.

# 5. Application Payload Common Header

Custom PSC ports 10, 11, and 12 use this application header.

| Offset | Size | Type | Name | Min | Max | Description |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | u8 | version | 1 | 1 | PSC application protocol version |
| 1 | 1 | u8 | opcode | command-specific | command-specific | Opcode |
| 2 | 2 | u16 | transaction_id | 0 | 65535 | Big-endian transaction ID |

# 6. Common Status Response

All status-only responses and all telemetry responses begin with this 6-byte status block.

| Offset | Size | Type | Name | Min | Max | Description |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | u8 | version | 1 | 1 | Echo protocol version |
| 1 | 1 | u8 | opcode | request opcode | request opcode | Echo opcode |
| 2 | 2 | u16 | transaction_id | 0 | 65535 | Echo transaction ID |
| 4 | 1 | u8 | status | 0 | 255 | Status code |
| 5 | 1 | u8 | detail | 0 | 255 | Error-specific detail |

| Status | Name | Meaning |
| --- | --- | --- |
| 0 | OK | Request accepted/completed |
| 1 | BAD_VERSION | Unsupported app protocol version |
| 2 | BAD_LENGTH | Payload length mismatch |
| 3 | BAD_OPCODE | Opcode not valid for the port |
| 4 | INVALID_ARGUMENT | Parameter out of accepted range |
| 5 | INVALID_STATE | Current PSC state does not allow command |
| 6 | APPLY_FAILED | Command decoded but failed while applying |
| 7 | INTERNAL_ERROR | Internal snapshot/encoding failure |
| 8 | BUSY | Reserved/available status |
| 255 | DROP | Internal drop status; normally no response |

# 7. Modes And Command Permissions

| Value | Mode |
| --- | --- |
| 0 | init_mode |
| 1 | normal_mode |
| 2 | run_mode |
| 3 | diagnostic_mode |

| TC | Accepted In | Notes |
| --- | --- | --- |
| SET_OUTPUTS | all modes | LPV/Heater/SP are applied in all modes. HPV bits are applied only in diagnostic_mode. |
| SET_LPV_OUTPUTS | all modes | LPV only. Successful response is broadcast to CSP address 0x1F. |
| SET_MODE | all modes | Requests mode 0..3. |
| THRUSTER_START | normal_mode only | If accepted, PSC enters run_mode; after sequence completes, PSC returns to normal_mode. |
| PAR_START | normal_mode only | PAR routine continues after start until PAR_STOP. |
| PAR_STOP | all modes | Stops PAR and closes PAR valves. |
| SIM_START | normal_mode, diagnostic_mode | Enables sensor override. SIM is not a state-machine mode. |
| SET_SIM_SENSOR_VALUES | after SIM_START | Updates override sensor values. |
| SIM_STOP | all modes | Disables sensor override. |
| FAULT_CLEAR | all modes | Clears latched thruster fault flags. |

# 8. Telecommand Definitions

### 8.1 SET_OUTPUTS

| Item | Value |
| --- | --- |
| Port | 10 |
| Opcode | 0x01 |
| Request length | 10 bytes |
| Success response | Sensor Snapshot TM, 126 bytes |

| Offset | Size | Type | Name | Min | Max | Unit/Encoding |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | u8 | version | 1 | 1 | fixed |
| 1 | 1 | u8 | opcode | 0x01 | 0x01 | fixed |
| 2 | 2 | u16 | transaction_id | 0 | 65535 | BE |
| 4 | 2 | u16 | lpv_on_mask | 0x0000 | 0x0FFF | bit0=SV-R01/LPV1 ... bit11=SV-R12/LPV12 |
| 6 | 1 | u8 | hpv_on_mask | 0x00 | 0xFF | bit0=SV-O1/HPV1 ... bit7=SPARE/HPV8 |
| 7 | 1 | u8 | heater_on_mask | 0x0 | 0xF | bit0=HTR1 ... bit3=HTR4 |
| 8 | 1 | u8 | spark_on | 0 | 1 | 0=OFF, 1=ON |
| 9 | 1 | u8 | reserved | 0 | 0 | fixed |

HPV bits are ignored unless PSC is in diagnostic_mode.

### 8.2 SET_MODE

| Item | Value |
| --- | --- |
| Port | 10 |
| Opcode | 0x02 |
| Request length | 5 bytes |
| Success response | Status, 6 bytes |

| Offset | Size | Type | Name | Min | Max | Unit/Encoding |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | u8 | version | 1 | 1 | fixed |
| 1 | 1 | u8 | opcode | 0x02 | 0x02 | fixed |
| 2 | 2 | u16 | transaction_id | 0 | 65535 | BE |
| 4 | 1 | u8 | mode | 0 | 3 | see mode table |

### 8.3 THRUSTER_START

| Item | Value |
| --- | --- |
| Port | 10 |
| Opcode | 0x03 |
| Request length | 32 bytes |
| Success response | Status, 6 bytes |
| Accepted mode | normal_mode only |

| Offset | Size | Type | Name | Min | Max | Unit/Encoding |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | u8 | version | 1 | 1 | fixed |
| 1 | 1 | u8 | opcode | 0x03 | 0x03 | fixed |
| 2 | 2 | u16 | transaction_id | 0 | 65535 | BE |
| 4 | 4 | u32 | burn_time_ms | 1 | 4294967295 | ms |
| 8 | 4 | u32 | sv_o3_open_delay_ms | 0 | 4294967295 | ms after command accept |
| 12 | 4 | u32 | sv_f3_open_delay_ms | 0 | 4294967295 | ms after command accept |
| 16 | 4 | u32 | spark_on_delay_ms | 0 | 4294967295 | ms after command accept |
| 20 | 4 | u32 | spark_on_duration_ms | 0 | 4294967295 | ms |
| 24 | 4 | u32 | sv_o3_close_delay_ms | 0 | 4294967295 | ms after burn end |
| 28 | 4 | u32 | sv_f3_close_delay_ms | 0 | 4294967295 | ms after burn end |

Current sequence valves:

| Function | Physical valve |
| --- | --- |
| Oxidizer thruster valve | SV-O3 / HPV3 |
| Fuel thruster valve | SV-F3 / HPV7 |
| Ignition | Spark Plug / Heater05 |

Current firmware validates only burn_time_ms != 0. Other timing fields are accepted over the full u32 range.

### 8.4 PAR_START

| Item | Value |
| --- | --- |
| Port | 10 |
| Opcode | 0x04 |
| Request length | 5 bytes |
| Success response | Status, 6 bytes |
| Accepted mode | normal_mode only |

| Offset | Size | Type | Name | Min | Max | Unit/Encoding |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | u8 | version | 1 | 1 | fixed |
| 1 | 1 | u8 | opcode | 0x04 | 0x04 | fixed |
| 2 | 2 | u16 | transaction_id | 0 | 65535 | BE |
| 4 | 1 | u8 | selector | 0 | 3 | bit field |

Selector:

| Bit | 0 | 1 |
| --- | --- | --- |
| bit0 | Ox pressure sensor PT-O3 | Ox pressure sensor PT-O4 |
| bit1 | Fuel pressure sensor PT-F3 | Fuel pressure sensor PT-F4 |

### 8.5 PAR_STOP

| Item | Value |
| --- | --- |
| Port | 10 |
| Opcode | 0x05 |
| Request length | 4 bytes |
| Success response | Status, 6 bytes |
| Accepted mode | all modes |

Payload is only the common request header.

### 8.6 SET_LPV_OUTPUTS

| Item | Value |
| --- | --- |
| Port | 10 |
| Opcode | 0x06 |
| Request length | 6 bytes |
| Success response | Broadcast Sensor Snapshot TM, 126 bytes, destination address 0x1F |

| Offset | Size | Type | Name | Min | Max | Unit/Encoding |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | u8 | version | 1 | 1 | fixed |
| 1 | 1 | u8 | opcode | 0x06 | 0x06 | fixed |
| 2 | 2 | u16 | transaction_id | 0 | 65535 | BE |
| 4 | 2 | u16 | lpv_on_mask | 0x0000 | 0x0FFF | bit0=SV-R01/LPV1 ... bit11=SV-R12/LPV12 |

### 8.7 SET_SIM_SENSOR_VALUES

| Item | Value |
| --- | --- |
| Port | 10 |
| Opcode | 0x07 |
| Request length | 60 bytes |
| Success response | Sensor Snapshot TM, 126 bytes |
| Precondition | SIM_START accepted |

| Offset | Size | Type | Name | Min | Max | Unit/Encoding |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | u8 | version | 1 | 1 | fixed |
| 1 | 1 | u8 | opcode | 0x07 | 0x07 | fixed |
| 2 | 2 | u16 | transaction_id | 0 | 65535 | BE |
| 4 | 36 | i32[9] | pt_millibar | -2147483648 | 2147483647 | mbar |
| 40 | 20 | i32[5] | tc_millikelvin | -2147483648 | 2147483647 | mK |

TC override validity is set only when each tc_millikelvin is in 1..2500000 mK.

### 8.8 SIM_START

| Item | Value |
| --- | --- |
| Port | 10 |
| Opcode | 0x08 |
| Request length | 4 bytes |
| Success response | Status, 6 bytes |
| Accepted modes | normal_mode, diagnostic_mode |

Payload is only the common request header. On start, override values are initialized from the most recent real sensor values.

### 8.9 SIM_STOP

| Item | Value |
| --- | --- |
| Port | 10 |
| Opcode | 0x09 |
| Request length | 4 bytes |
| Success response | Status, 6 bytes |
| Accepted mode | all modes |

Payload is only the common request header.

### 8.10 FAULT_CLEAR

| Item | Value |
| --- | --- |
| Port | 10 |
| Opcode | 0x0A |
| Request length | 4 bytes |
| Success response | Status, 6 bytes |
| Accepted mode | all modes |

Payload is only the common request header. This clears latched thruster fault flags.

# 9. Telemetry Definitions

### 9.1 GET_SENSOR_SNAPSHOT

| Item | Value |
| --- | --- |
| Port | 11 |
| Opcode | 0x01 |
| Request length | 4 bytes |
| Response length | 126 bytes |

Request payload is only the common request header.

Response:

| Offset | Size | Type | Name | Min | Max | Unit/Encoding |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 6 | status | status_block | - | - | common status response |
| 6 | 4 | u32 | sample_time_ms | 0 | 4294967295 | ms |
| 10 | 1 | u8 | current_mode | 0 | 3 | mode |
| 11 | 1 | u8 | requested_mode | 0 | 3 | mode |
| 12 | 2 | u16 | validity_mask | 0 | 65535 | bit mask |
| 14 | 36 | i32[9] | pt_millivolt | -2147483648 | 2147483647 | mV, pressure sensor input-side voltage after PSC internal calibration |
| 50 | 36 | i32[9] | pt_millibar | -2147483648 | 2147483647 | mbar |
| 86 | 20 | i32[5] | tc_microvolt | -2147483648 | 2147483647 | uV |
| 106 | 20 | i32[5] | tc_millikelvin | -2147483648 | 2147483647 | mK |

Pressure sensor TM values are transmitted after PSC-side divider compensation and per-channel calibration. OBC shall treat pt_millivolt and pt_millibar as calibrated telemetry values; calibration coefficients are PSC internal implementation data and are not part of this ICD.

Current validity mask:

| Bits | Meaning |
| --- | --- |
| bit0..bit8 | PT channels valid |
| bit9..bit13 | TC temperature channels valid |

### 9.2 GET_SOLVALVE_STATE

| Item | Value |
| --- | --- |
| Port | 11 |
| Opcode | 0x02 |
| Request length | 4 bytes |
| Response length | 16 bytes |

Request payload is only the common request header.

Response:

| Offset | Size | Type | Name | Min | Max | Unit/Encoding |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 6 | status | status_block | - | - | common status response |
| 6 | 4 | u32 | sample_time_ms | 0 | 4294967295 | ms |
| 10 | 1 | u8 | current_mode | 0 | 3 | mode |
| 11 | 2 | u16 | lpv_on_mask | 0x0000 | 0x0FFF | LPV state |
| 13 | 1 | u8 | hpv_on_mask | 0x00 | 0xFF | HPV state |
| 14 | 1 | u8 | heater_on_mask | 0x0 | 0xF | Heater state |
| 15 | 1 | u8 | spark_on | 0 | 1 | Spark Plug state |

### 9.3 GET_HEALTH

| Item | Value |
| --- | --- |
| Port | 12 |
| Opcode | 0x01 |
| Request length | 4 bytes |
| Response length | 17 bytes |

Request payload is only the common request header.

Response:

| Offset | Size | Type | Name | Min | Max | Unit/Encoding |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 6 | status | status_block | - | - | common status response |
| 6 | 4 | u32 | uptime_ms | 0 | 4294967295 | RTOS tick count in ms-equivalent |
| 10 | 1 | u8 | current_mode | 0 | 3 | mode |
| 11 | 1 | u8 | link_state | 0 | 255 | RS485 link state enum |
| 12 | 1 | u8 | last_error | 0 | 255 | RS485 link last error enum |
| 13 | 4 | u32 | thruster_fault_flags | 0 | 4294967295 | bit flags |

GET_HEALTH intentionally excludes debug message queue and link counters.

# 10. CSP Standard Services

### 10.1 CSP_PING

| Item | Value |
| --- | --- |
| Port | 1 |
| App header | Not used |
| Suggested payload | ASCII PSC-PING, hex 50 53 43 2D 50 49 4E 47 |
| Response | Echo of request payload |

### 10.2 CSP_REBOOT

| Item | Value |
| --- | --- |
| Port | 4 |
| App header | Not used |
| Payload | u32 magic 0x80078007, big endian |
| Response | Do not expect an application response |

Accepted reboot request causes PSC to call NVIC_SystemReset().

# 11. Channel Mapping

### 11.1 Pressure Sensors

| Index | TM Label | Physical |
| --- | --- | --- |
| 0 | PT-O1 | PT1 |
| 1 | PT-O2 | PT2 |
| 2 | PT-O3 | PT3 |
| 3 | PT-O4 | PT4 |
| 4 | PT-F1 | PT5 |
| 5 | PT-F2 | PT6 |
| 6 | PT-F3 | PT7 |
| 7 | PT-F4 | PT8 |
| 8 | PT-C1 | PT9 |

Pressure conversion:

| Sensor | 0 bar | Full scale | Full-scale pressure |
| --- | --- | --- | --- |
| PT-O1..O4, PT-F1..F4 | 0.5 V | 4.5 V | 100 bar |
| PT-C1 | 0.5 V | 4.5 V | 16 bar |

The firmware compensates the pressure ADC divider using:

gain = (20000 + 78700/2) / (78700/2)

### 11.2 Temperature Sensors

| Index | TM Label | Electrical |
| --- | --- | --- |
| 0 | TC-O1 | TC1 AIN0/1 |
| 1 | TC-O2 | TC2 AIN2/3 |
| 2 | TC-F1 | TC3 AIN4/5 |
| 3 | TC-C1 | TC4 AIN6/7 |
| 4 | CJC1 10k NTC | AIN8/9 |

TC valid converted range is 0 K < T <= 2500 K.

### 11.3 Solenoid Valves

| Bit | LPV Mask |
| --- | --- |
| 0..11 | SV-R01..SV-R12 / LPV1..LPV12 |

| Bit | HPV Mask |
| --- | --- |
| 0 | SV-O1 / HPV1 |
| 1 | SV-O2 / HPV2 |
| 2 | SV-O3 / HPV3 |
| 3 | SPARE / HPV4 |
| 4 | SV-F1 / HPV5 |
| 5 | SV-F2 / HPV6 |
| 6 | SV-F3 / HPV7 |
| 7 | SPARE / HPV8 |

| Bit | Heater Mask |
| --- | --- |
| 0 | HTR1 |
| 1 | HTR2 |
| 2 | HTR3 |
| 3 | HTR4 |

Spark Plug is a separate u8 field: 0=OFF, 1=ON.

# 12. Fault Flags

| Bit Mask | Name | Meaning |
| --- | --- | --- |
| 0x00000000 | THRUSTER_FAULT_NONE | No latched thruster fault |
| 0x00000001 | THRUSTER_FAULT_PT_C1_HH | PT-C1 above HH threshold |
| 0x00000002 | THRUSTER_FAULT_PT_C1_LL | PT-C1 below LL threshold |
| 0x00000004 | THRUSTER_FAULT_TT_C1_HH | TT-C1 above HH threshold |
| 0x00000008 | THRUSTER_FAULT_PT_C1_INVALID | PT-C1 invalid/out of plausible range |
| 0x00000010 | THRUSTER_FAULT_TT_C1_INVALID | TT-C1 invalid/out of plausible range |

Current run-monitor thresholds:

| Tag | Start | End | HH | LL | Valid range |
| --- | --- | --- | --- | --- | --- |
| PT-C1 | 3250 ms | 20250 ms | 7800 mbar | 3000 mbar | -500..17000 mbar |
| TT-C1 | 3250 ms | 20250 ms | 1200000 mK | none | 1..2500000 mK |

Fault flags are latched until FAULT_CLEAR is accepted.

# 13. Example Frames

Examples use:

| Field | Value |
| --- | --- |
| Priority | 2 |
| Source address | OBC 0x0A |
| Destination address | PSC 0x10 |
| Flags | 0x00 |

### 13.1 GET_HEALTH

Request app payload:

01 01 00 01

Frame with source port 14, destination port 12:

c0 00 95 03 0e 00 01 01 00 01 c2 08 f0 02 c0

### 13.2 GET_SENSOR_SNAPSHOT

Request app payload:

01 01 00 02

Frame with source port 15, destination port 11:

c0 00 95 02 cf 00 01 01 00 02 d1 58 03 f6 c0

### 13.3 SET_LPV_OUTPUTS All Off

Request app payload:

01 06 00 03 00 00

Frame with source port 16, destination port 10:

c0 00 95 02 90 00 01 06 00 03 00 00 c2 53 b6 08 c0

### 13.4 CSP_PING

Payload:

50 53 43 2d 50 49 4e 47

Frame with source port 17, destination port 1:

c0 00 95 00 51 00 50 53 43 2d 50 49 4e 47 f6 5e e8 02 c0

### 13.5 CSP_REBOOT

Payload:

80 07 80 07

Frame with source port 18, destination port 4:

c0 00 95 01 12 00 80 07 80 07 41 3e 78 83 c0

# 14. OBC Bring-Up Checklist

Open RS485 UART at 921600 8N1.

Send CSP_PING to address 0x10, port 1; verify echo payload.

Send GET_HEALTH; verify 17-byte app response and matching transaction ID.

Send GET_SENSOR_SNAPSHOT; verify 126-byte app response.

Send GET_SOLVALVE_STATE; verify 16-byte app response.

In normal_mode, test SET_LPV_OUTPUTS.

In diagnostic_mode, test HPV bits through SET_OUTPUTS.

Verify rejected mode cases return INVALID_STATE.

Verify THRUSTER_START enters run_mode and later returns to normal_mode.

Verify fault flags remain latched until FAULT_CLEAR.